| Course Name: | Digital Design Laboratory | Semester: | III |
| --- | --- | --- | --- |
| Date of Performance: | 09/ 09 / 24 | Batch No: | C3 |
| Faculty Name: | Bharathi Narayan | Roll No: | 16010123217 |
| Faculty Sign & Date: |  | Grade/Marks: | ___/25 |

Experiment No: 7

Title: Asynchronous Counter

Aim and Objective of the Experiment:

To design and implement 3 bit Asynchronous up counter using JK Flip Flop

COs to be achieved:

CO3: Design synchronous and asynchronous sequential circuits.

Tools used:

Trainer kits

Theory:

Circuit diagram of 3 bit Asynchronous Up counter using JK FF (IC 7476) Pin diagram of JK FF (IC 7476)

Implementation Details

Procedure - Locate IC 7476 JK FF on Digital trainer kit - Apply various inputs to appropriate pins as per the circuit diagram of the 3 bit Asynchronous up counter with reference to the pin configuration of the IC. - Make sure of Reset and Clear Pins connections with reference to data sheet information. - Connect a pulsar switch to the clock input. - Verify the working and prepare a truth table.

Post Lab Subjective/Objective type Questions:

- How JK FF need to be configured to use for counter operation? Ans. To configure a JK flip-flop (FF) for counter operation: - Clock input: Provide the clock signal to trigger the flip-flop. - Use multiple JK FFs: For counters (e.g., binary counters), connect the Q output of one flip-flop to the clock input of the next flip-flop in the series. - J = K = 1: This makes the JK flip-flop toggle its state on every clock pulse. - What changes are required to use the same counter as 3 bit asynchronous down counter? Ans. To use the same counter as a 3-bit asynchronous down counter, follow these steps: - Invert the Q output: Connect the inverted output (Q') of each JK flip-flop to the clock input of the next flip-flop, instead of the Q output. - J = K = 1: Keep J = K = 1 to maintain the toggling behavior on each clock pulse. - Clock input: Provide the clock signal to the first flip-flop, which triggers the rest in sequence. - Draw the timing diagram of 3 bit Asynchronous up counter. Ans. - What is mod n concept used in counters? Ans. Counters whose values “wrap around” back to zero when they reach the value n are called modulo-n counters. The most common example is a modulo-10 counter that counts from 0 up to 9 and then “wraps around” back to 0. - For Mod-5 counter how many JK FFs are required? Ans. The number of flip flops required for mod 5 is 3 because it is in between mod 4 and mod 8.

Conclusion:

By performing this experiment I learnt how to use JK flip flops

Signature of faculty in-charge with Date: