| Course Name: | Digital Design Laboratory | Semester: | III |
| --- | --- | --- | --- |
| Date of Performance: | 21 / 08 / 2023 | Batch No: | C-2 |
| Faculty Name: |  | Roll No: | 16010122267 |
| Faculty Sign & Date: |  | Grade/Marks: | ___/25 |

Experiment No: 4

Title: 4-bit magnitude comparator

Aim and Objective of the Experiment:

To design a 2-bit comparator using logic gates and verify 4-bit magnitude comparator using IC 7485

COs to be achieved:

CO2: Use different minimization technique and solve combinational circuits.

Tools used:

Trainer kits

Theory:

Comparator: The comparison of two numbers is an operator that determines one number is greater than, less than (or) equal to the other number. A magnitude comparator is a combinational circuit that compares two numbers A and B and determines their relative magnitude. The outcome of the comparator is specified by three binary variables that indicate whether A>B, A=B (or) A<B. Two Bit Magnitude Comparator Implementation Details: Truth Table from the Truth Table: INPUT; OUTPUT. A1; A0; B1; B0; A<B; A=B; A>B. 0; 0; 0; 0; 0; 1; 0. 0; 0; 0; 1; 1; 0; 0. 0; 0; 1; 0; 1; 0; 0. 0; 0; 1; 1; 1; 0; 0. 0; 1; 0; 0; 0; 0; 1. 0; 1; 0; 1; 0; 1; 0. 0; 1; 1; 0; 1; 0; 0. 0; 1; 1; 1; 1; 0; 0. 1; 0; 0; 0; 0; 0; 1. 1; 0; 0; 1; 0; 0; 1. 1; 0; 1; 0; 0; 1; 0. 1; 0; 1; 1; 1; 0; 0. 1; 1; 0; 0; 0; 0; 1. 1; 1; 0; 1; 0; 0; 1. 1; 1; 1; 0; 0; 0; 1. 1; 1; 1; 1; 0; 1; 0. Logic Diagram of 2-bit Comparator Four Bit Magnitude Comparator Implementation Details Pin Diagram of IC 7485 Logic Diagram of IC 7485 Comparing Table

Implementation Details

Procedure: - Locate the IC 7485 on the trainer kit. - Connect 1st input no. to A3-A0 input slot and 2nd to B3-B0. - Connect the output YA>B, YA<B and YA=B to the output indicators. - Switch ON the power supply and monitor the output for various input combinations.

Post Lab Subjective/Objective type Questions:

- Give some applications of magnitude comparator. Ans: Some applications of magnitude comparator are: - - CPU and MCU Operations: Magnitude comparators find applications within Central Processing Units (CPUs) and Microcontrollers (MCUs) where they are essential for various data comparison tasks. - Control Systems: They are employed in control applications where binary representations of physical variables such as temperature, position, etc., need to be compared with a predefined reference value to make control decisions. - Process Control: Magnitude comparators play a crucial role in process control systems, ensuring that parameters are within specified limits and triggering actions based on these comparisons. - Servo Motor Control: In applications involving servo motors, magnitude comparators are used to compare desired and actual positions, enabling precise motor control. - Security and Authentication: They are utilized in password verification and biometric authentication systems to determine if input data matches stored reference values. - Address Decoding in Computers: In computer architecture, magnitude comparators are used in address decoding circuits, aiding in the selection of specific memory locations or peripherals based on the address provided. - Explain with the help of the logic diagram how an 8-bit comparator can be implemented using IC 7485. An 8-bit comparator can be implemented by cascading of two 4-bit comparators (IC 7485). The outputs of the lower-order comparator are connected to the corresponding cascading inputs of the higher-order comparator. In the lower order comparator, the cascading input (A=B) needs to be connected HIGH, and A, B needs to be connected to LOW. The result of the 8-bit comparator is the output of the higher-order comparator.

Conclusion:

Thus, in this experiment, we learned about binary comparators and how to implement them using IC 7485. We learned about 1-bit, 2-bit, 4-bit and 8-bit comparators. We also learned how to build an 8-bit comparator using two 4-bits comparators.

Signature of faculty in-charge with Date: